5.2 Linear models: Boundary Value Problems

Another example where linear DEs come up

--------------------------------------------------

Deflection of a beam

Assume a beam of length L with uniform cross sections

Axis of symmetry

In real life it bends

y(x) = distance it sags

deflection or elastic curve

Bottom line:

EI y(4) = w(x)  where  E is Young’s modulus of elasticity of the material, I is moment of inertia of a cross section and w(x) is weight per unit length  at x

Discusssion:

Moment: used to describe a bending force on a beam

Bending moment M(x) is

- related to the load per unit length by  M” = w(x)

- proportional the the curvature of the elastic curve:  M(x) = EIκ

Curvature κ = y”/[1+(y’)2] 3/2    (does anyone remember this?)

When deflectinis small y’ ~ 1

κ ~ y”

M(x) = EIκ

M(x) = Eiy”

M” = w(x) -> (Eiy”)” = w(x)

Resulting DE:

EI y(4) = w(x)

Boundary conditions determined by how the ends are supported.

Example: a beam embedded at both ends (ends are rigid). Assume a uniform weight

w(x) = w0 per unit length

x(0)= x(L) =x’(0)= x’(L) = 0

(instantaneous change in x = 0 because of rigidity)

EIx(4) = w0

Homogeneous: y(x) = c1 + c2 x + c3 x2+ c3 x3

yp = A x4

Plug in DE: AEi(4)(3)(2) = w0

A = w0/24EI

General solution:

y(x) = c1 + c2 x + c3 x2+ c4 x3 + w0/24EI x4

x(0)=x’(0) = 0 give c1 = c2 =0

y(x) = c3 x2+ c4 x3 + w0/24EI x4

x(L)=x’(L) = 0 … c3  = w0L2/24EI, c4  =- w0L2/12EI

y(x) = w0L2/24EI x2- w0L2/12EI x3 + w0/24EI x4

note y”(0) = 2w0L2/24EI   (slope is changing)

How would this change if the beam hung free at L?

y(0) = y’(0) = 0, y”(L)= y”’(L) = 0

bending moment is 0, shear force is 0

You don’t know what y(L) or Y’(L) is but assume nothing contributes to a change in y’ or y’’

How would this change if the beam were simply supported?

Remove y’() = 0, add y”()= 0